Towards Sustainable Processing Of Municipal Household Organic Waste: The Role Of Energy Mix Grids

Christian Ottinia,b, Gwenola Yannou a,b, Sandra Domenek a,b and Felipe Buendiaa,b*

a Université Paris-Saclay, INRAE, AgroParisTech, UMR SayFood, 91120 Palaiseau, France

b Fondation AgroParisTech, Chaire CoPack, 91120 Palaiseau, France

* Corresponding Author: felipe.buendia-kandia@inrae.fr

Supplementary material

energy grid mix

The energetic mix of France 2022 and 2030 are detailed in the following tables (S1-2) and they were retrieved from the French strategic plan for energy and climate [1]. For the average EU energy grid the data have been collected from the internation energy agency (IEA)[2] table S3.

Table S1: Energy mix France 2022

| Technology | In % on the total |
| --- | --- |
| Nuclear | 17.6 |
| Oil | 37.9 |
| Natural gas | 25.1 |
| Coal | 0.9 |
| Renewable Heat* | 11.5 |
| Hydro-electric | 2.7 |
| Eolic onshore | 2.5 |
| Eolic offshore | 0.1 |
| Photovoltaic | 1.2 |
| Biogas | 0.7 |

Table S1.1: Renewable Heat share France 2022

| Technology | In % on Renewable Heat |
| --- | --- |
| Biomass | 67.3 |
| Geothermic | 3.4 |
| Heat pumps | 21.4 |
| Thermic photovoltaic | 0.7 |
| Waste heat recovery | 3 |
| Biogas | 4.1 |
| Waste incineration | 0.11 |

Table S2: Energy mix grid France 2030

| Technology | In % on the total |
| --- | --- |
| Nuclear | 25.1 |
| Oil | 23.4 |
| Natural gas | 16.8 |
| Coal | 0 |
| Renewable Heat* | 18.8 |
| Hydro-electric | 3.4 |
| Eolic onshore | 4 |
| Eolic offshore | 0.9 |
| Photovoltaic | 4.1 |
| Biogas | 3.1 |

In tab. 2 Hydrogen production have been omitted

Table S2.1: Renewable Heat share France 2030

| Technology | In % on Renewable Heat |
| --- | --- |
| Biomass | 45.1 |
| Geothermic | 5.4 |
| Heat pumps | 25 |
| Thermic photovoltaic | 2 |
| Waste heat recovery | 6.7 |
| Biogas | 12.5 |
| Waste incineration | 3.4 |

Table S3: Average energy mix grid EU 2022

| Technology | In % on the total |
| --- | --- |
| Nuclear | 10.8 |
| Oil | 32.3 |
| Natural gas | 24.7 |
| Coal | 13.5 |
| Renewables exc. Hydro-electric | 5.4 |
| Hydro-electric | 2.6 |
| Biofuel and waste | 10.7 |

Simulation of the incineration process

Table S4: State of art description of the incineration technologies used in Paris region and the amount of waste treated annually. Extracted from [3]

| Incineration unit | Waste treated annually (t) | Steam sold to CPCU1 (MWh) | Electricity sold to EDF2(MWh) | Electricity production efficiency3 (%) | Heat production efficiency3 (%) |
| --- | --- | --- | --- | --- | --- |
| Isseane | 506889 | 987237 | 61943 | 5 | 56 |
| Ivry/ Paris XIII | 544144 | 634136 | 57905 | 4.4 | 48.1 |
| L’étoile verte | 495918 | 825690 | 4585 | 0.4 | 68.8 |
| Total | 1546951 | 2157063 | 124433 | 3.3 | 57.6 |

1 Parisian Urban Heating Company (Compagnie Parisienne de Chauffage Urbain), Parisian distributor for Heat.

2 Electricity of France (Électricité de France), the major producer and distributor of energy in France.

3 Calculated as total energy in waste/ electricity or heat produced. The LHV of OFMSW was found to be 2.42 MWh/t.

Table S4 contains the description of the actual incineration plants still in use in Paris as for the year 2023, with the quantity of waste treated annually, the steam and electricity sold and the calculated efficiencies both electrical and thermal. And tab. S4 the characterization of the OFMSW found in Paris.

Table S5: Qualitative analysis of the OFMSW produced in the Parisian Region. [4]

| Element | Symbol | Units | Organic waste |
| --- | --- | --- | --- |
| lower heating value | Hu | MJ/kg | 5.81E+00 |
| Water content | H2O | kg/kg waste | 6.00E-01 |
| Oxygen (without O from H2O) | O | kg/kg waste | 1.26E-01 |
| Hydrogen (without H from H2O) | H | kg/kg waste | 2.52E-02 |
| Carbon | C | kg/kg waste | 1.64E-01 |
| Sulphur | S | kg/kg waste | 1.48E-03 |
| Nitrogen | N | kg/kg waste | 1.12E-02 |
| Phosphor | P | kg/kg waste | 1.40E-03 |
| Boron | B | kg/kg waste |  |
| Chlorine | Cl | kg/kg waste | 2.34E-03 |
| Bromine | Br | kg/kg waste | 3.36E-05 |
| Fluorine | F | kg/kg waste | 1.18E-05 |
| Iodine | I | kg/kg waste |  |
| Silver | Ag | kg/kg waste | 0.00E+00 |
| Arsenic | As | kg/kg waste | 8.00E-08 |
| Barium | Ba | kg/kg waste | 1.94E-04 |
| Cadmium | Cd | kg/kg waste | 1.00E-07 |
| Cobalt | Co | kg/kg waste | 7.50E-06 |
| Chromium | Cr | kg/kg waste | 4.28E-06 |
| Copper | Cu | kg/kg waste | 8.80E-06 |
| Mercury | Hg | kg/kg waste | 1.24E-07 |
| Manganese | Mn | kg/kg waste | 4.73E-05 |
| Molybdenum | Mo | kg/kg waste | 4.13E-07 |
| Nickel | Ni | kg/kg waste | 2.16E-06 |
| Lead | Pb | kg/kg waste | 3.04E-05 |
| Antimony | Sb | kg/kg waste | 1.52E-06 |
| Selenium | Se | kg/kg waste | 1.60E-08 |
| Tin | Sn | kg/kg waste | 5.56E-06 |
| Vanadium | V | kg/kg waste | 6.14E-06 |
| Zinc | Zn | kg/kg waste | 1.64E-04 |
| Beryllium | Be | kg/kg waste |  |
| Scandium | Sc | kg/kg waste |  |
| Strontium | Sr | kg/kg waste |  |
| Titanium | Ti | kg/kg waste |  |
| Thallium | Tl | kg/kg waste |  |
| Tungsten | W | kg/kg waste |  |
| Silicon | Si | kg/kg waste | 4.00E-02 |
| Iron | Fe | kg/kg waste | 1.56E-03 |
| Calcium | Ca | kg/kg waste | 1.56E-02 |
| Aluminium | Al | kg/kg waste | 3.25E-03 |
| Potassium | K | kg/kg waste | 3.21E-03 |
| Magnesium | Mg | kg/kg waste | 2.82E-03 |
| Sodium | Na | kg/kg waste | 1.49E-03 |

Simulation of the Composting process

For the modelling and simulation of the composting process we detailed the compost produced as shown in tab. S6.

Table S6: Detailed Characterization of Compost as from [5]

| Parameter | Unit | Value |
| --- | --- | --- |
| TS | % ww | 50.4 |
| VS | % TS | 30.9 |
| Ash | % TS | 69.1 |
| S | % TS | 0.45 |
| Tot-Cl | % TS | 0.08 |
| F | % TS | <0.01 |
| Tot-C | % TS | 19.1 |
| H | % TS | 2.1 |
| Tot-N | % TS | 1.2 |
| O | % TS | 7.2 |
| C:N |  | 15.9 |
| Cal. value | MJ/kg TS | 6.6 |
| Cal. value | MJ/kg ww | 2.8 |
| Density | kg/m³ | 726 |
| As | mg/kg TS | 6.5 |
| Ba | mg/kg TS | 320 |
| Be | mg/kg TS | 0.73 |
| Cd | mg/kg TS | 0.62 |
| Co | mg/kg TS | 3.6 |
| Cr | mg/kg TS | 18 |
| Cu | mg/kg TS | 135 |
| Hg | mg/kg TS | 0.53 |
| Mo | mg/kg TS | <4.0 |
| Nb | mg/kg TS | <4.0 |
| Ni | mg/kg TS | 13.5 |
| Pb | mg/kg TS | 28 |
| Sb | mg/kg TS | 0.23 |
| Sc | mg/kg TS | 1.8 |
| Sn | mg/kg TS | <20 |
| Sr | mg/kg TS | 279 |
| V | mg/kg TS | 24 |
| W | mg/kg TS | <40 |
| Y | mg/kg TS | 8.0 |
| Zn | mg/kg TS | 395 |
| Zr | mg/kg TS | 124 |
| Si | g/kg TS | 187 |
| Al | g/kg TS | 22 |
| Ca | g/kg TS | 81 |
| Fe | g/kg TS | 18 |
| K | g/kg TS | 11 |
| Mg | g/kg TS | 3.7 |
| Mn | g/kg TS | 0.50 |
| Na | g/kg TS | 4.2 |
| P | g/kg TS | 9.3 |
| Ti | g/kg TS | 1.4 |

The amount of credited mineral fertilizer substitution has been calculated with the following equations (S1-3)

Nfert=(Cm∙TS∙Tot-N)∙BioN	     eq. S1

Pfert=Cm∙TS∙P∙fc1000			      eq. S2

Kfert=Cm∙TS∙K∙fc11000	                                   eq. S3

With Nfert, Pfert and Kfert namely the mass in kg of mineral nitrogen (calcium ammonium nitrate), phosphate (diammonium phosphate) and potassium fertilizer substituted. Cm compost mass in kg. fc the correlation factor between P and P2O5 and fc1 between K and K2O, namely 0.22 and 0.42. This correlation factor is equal to the ratio of P and K in the two molecules. BioN the bioavailability of nitrogen present in the compost, set to 30% P and K are set to a bioavailability of 100%.

Simulation of the Anaerobic digestion-composting process

The anaerobic digestion digestate has been detailed in tab. S7

Table S7: Inventory for the digestate as from [6]

| Parameter | Unit | Value |
| --- | --- | --- |
| Digestate dry solid (DS) fraction (%) | % | 4.5 |
| Tot-N | % of DS | 15 |
| Tot-P | % of DS | 0.7 |
| Tot-K | % of DS | 4.7 |
| Nitrogen fertiliser displaced | Bioavailability (%) | 40 |
| Potassium fertiliser displaced | Bioavailability (%) | 100 |
| Phosphate fertiliser displaced | Bioavailability (%) | 100 |
| Nitrous oxide emission air from application | % total N emitted as N2O-N | 0.45 |
| Nitrate leaching to soil from application | % total N emitted as Nitrate-N | 15 |

The substituted fertiliser has been calculated using the eq. (S4-6)

Nfert=(Dm∙DS∙Tot-N)∙BioN	eq. S4

Pfert=(Dm∙DS∙Tot-P)∙fc	  eq. S5

Kfert=(Dm∙DS∙Tot-K)∙fc1	 eq. S6

With Nfert, Pfert and Kfert namely the mass in kg of mineral nitrogen (calcium ammonium nitrate), phosphate (diammonium phosphate) and potassium fertilizer substituted. Dm digestate mass in kg. fc the correlation factor between P and P2O5 and fc1 between K and K2O, namely 0.22 and 0.42. This correlation factor is equal to the ratio of P and K in the two molecules. BioN the bioavailability of nitrogen present in the compost, P and K are set to a bioavailability of 100%.

For the emission linked to the application we simply multiply the values in the tab. 6 for the total nitrogen present in the digestate.

Environmental assessment results

In Table S8 and S9 we can find, namely the environmental midpoint impact of the three technologies, France vs EU energy grids and France today vs 2030 energy grids.

Table S8:  Tabular data of the LCA impacts of the various technologies with credits studied in this article regarding France and average EU energy mix grid.

Table S9:  Tabular data of the LCA impacts of the various technologies with credits studied in this article regarding France today and 2030 energy mix grid.

References

Stratégie française pour l’énergie et le climat. https://www.ecologie.gouv.fr/sites/default/files/documents/23242_Strategie-energie-climat.pdf

International Energy Agency, Energy supply EU. https://www.iea.org/regions/europe/energy-mix

Rapport d’activité 2023.https://www.syctom-paris.fr/fileadmin/user_upload/37685_Syctom_RA_2023_et_Financier.pdf

Kumar A, Samadder SR. Development of lower heating value prediction models and estimation of energy recovery potential of municipal solid waste and rdf incineration. Energy 274:127273 (2023). https://doi.org/10.1016/j.energy.2023.127273.

Beylot A, Muller S, Descat M, Ménard Y, Villeneuve J. Life cycle assessment of the french municipal solid waste incineration sector. Waste Management 80:144–153 (2018). https://doi.org/10.1016/j.wasman.2018.08.037.

Boldrin A, Hartling KR, Laugen M, Christensen TH. Environmental inventory modelling of the use of compost and peat in growth media preparation. Resources, Conservation and Recycling 54:1250–1260 (2010). https://doi.org/10.1016/j.resconrec.2010.04.003.