Digital Supplementary Information:

Sustainable production of L-lactic acid from lignocellulosic biomass using an alternative buffer system: Process development and techno-economic and environmental analysis

Donggeun Kang, Donghyeon Kim, Dongin Jung, Siuk Roh, and Jiyong Kim*

a School of Chemical Engineering, Sungkyunkwan University, Suwon 16419, Republic of Korea

* Corresponding author. E-mail address: jiyongkim@skku.edu (J. Kim).

Table S1. Major reactions and process conditions

| Area | Reaction | T (°C) | P (atm) | Time | yield(%) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
| A200 Dilute acid pretreatment | (Glucan)n + n H2O → n Glucose | 150 | 5 | 5 min | 9.9 | [1] |
|  | (Xylan)n + n H2O → n Xylose |  |  |  | 90.0 |  |
| A200 Enzyme hydrolysis | (Glucan)n + n H2O → n Glucose | 48 | 1 | 84 hr | 90.0 | [1] |
| A300 L-LA fermentationa,c | Glucose → 2 L-LA | 50 | 1 | 30 hr | 95.0 | [2] |
|  | 3 Xylose → 5 L-LA |  |  |  | 95.0 |  |
| A300 L-LA fermentationb,c | Glucose → 2 L-LA | 42 | 1 | 72 hr | 80.0 | [3] |
|  | 3 Xylose → 5 L-LA |  |  |  |  |  |
| A600 KCl electrolysis | 2 H2O + 2 KCl → H2 + Cl2 + KOH | 90 | 1 | - | - | [4] |

a Data from fermentation using Ca(OH)2 as buffer

b Data from fermentation using KOH as buffer

c Overall yields are presented as combined sugar-to-L-LA conversion ratios to provide a unified metric for process efficiency evaluation

Table S2.  Median mass fraction (%) ± expanded uncertainty of biomass components found in NIST biomass reference material samples [5]

|  |  | Material | Constituent | (%) |
| --- | --- | --- | --- | --- |
|  |  | Eastern Cottonwood | Glucan | 46.1 |
|  |  |  | Xylan | 14.7 |
|  |  |  | Arabinan | 0.7 |
|  |  |  | Galactan | 0.7 |
|  |  |  | Mannan | 2.2 |
|  |  |  | Lignin | 28.0 |
|  |  |  | Acid | 4.0 |
|  |  |  | Ash | 1.1 |

[1]	Humbird, Davis, et al. Process design and economics for biochemical conversion of lignocellulosic biomass to ethanol: dilute-acid pretreatment and enzymatic hydrolysis of corn stover. No. NREL/TP-5100-47764. National Renewable Energy Lab.(NREL), Golden, CO (United States), (2011)

[2]	Ma, Kedong, et al. "Highly efficient production of optically pure L-lactic acid from corn stover hydrolysate by thermophilic Bacillus coagulans." Bioresource technology 219:114-122 (2016)

[3]	van der Pol, Edwin C., Gerrit Eggink, and Ruud A. Weusthuis. "Production of l (+)-lactic acid from acid pretreated sugarcane bagasse using Bacillus coagulans DSM2314 in a simultaneous saccharification and fermentation strategy." Biotechnology for biofuels 9:1-12 (2016)

[4]	Suhara, Manabu, and Yoshio Oda. "Electrolysis of aqueous solution of potassium chloride." U.S. Patent No. 4,261,803. (1981)

[5]	Templeton, David W., et al. "Compositional analysis of biomass reference materials: results from an interlaboratory study." Bioenergy research 9: 303-314 (2016)