# DIGITAL SUPPLEMENTARY MATERIAL

Table S1: PolyActive membrane permeance (Wang, 2020)

| Temperature [°C] | 50 | 30 | 25 |
| --- | --- | --- | --- |
| CO2 | 5 | 4.3 | 3 |
| H2O | 15 | 43.3 | 53 |
| CO2/N2 | 25 | 36 | 50 |
| O2/N2 | 2.8 | 2.8 | 2.8 |

The cost data for the PolyActive membrane are presented in Table S2 and Equation 1 can be applied to estimate the cost at a given capacity. Membrane and frame (Cframe0) costs of $50 and membrane area (A, m²)  are used with the frame cost equation is presented in Equation 2. For the membrane cost, a first-of-a-kind (FOAK) cost of $200 per m² is used (Zhai, 2019), as limited real-world data on membrane carbon capture units are publicly available.

CiCiref=SiSirefn                                                               (1)

Cframe=A20000.7×Cframe0                                       (2)

Table S2: PolyActive membrane cost data (US$2010)

| Equipment | Capacity Ciref | Cost Siref | Exponent n |
| --- | --- | --- | --- |
| Heat Exchanger | 80 m2 | 3.28 × 104 | 0.68 |
| Compressor | 250 kW | 9.84 × 104 | 0.46 |
| Pump | 4 kW | 9.84 × 103 | 0.55 |

Table S3: CapEx and OpEx estimation method presented in IEAGHG (2024)

| CapEx method | % |
| --- | --- |
| Equipment Purchase Cost (EPC) |  |
| Installation cost/ supporting facilities[*] |  |
| Total Equipment Cost (TEC) | Sum of above |
| Total Plant cost (TPC) | 130% TEC |
| Total Capital Requirement (TCR) = Total CapEx | 110% TPC |
| OpEx method |  |
| Fixed O&M Cost | 1% TCR |
| Variable O&M Cost | 2% TCR |

[*] The IEAGHG (2024) report provides only the Total Equipment Cost (TEC),  which includes the Equipment Purchase Cost (EPC) and installations/ supporting facilities, without specifying the assumptions for installation and supporting facilities. Therefore, in this work, assumptions from Turton et al., (2018) were applied, where supporting facilities accounts for 71.4% EPC and installation cost is 37% (EPC + 71.4 EPC).

Table S4: Small-scale MEA data points

| CO2 concentration [mol%] | Gas flow rates [Nm3/h] | Capture scale [kt/y] |
| --- | --- | --- |
| 5 | 12 910 | 10 |
|  | 40 345 | 31 |
|  | 80 690 | 62 |
|  | 127 490 | 99 |
| 11 | 18 559 | 33 |
|  | 40 345 | 73 |
| 13 | 45 187 | 91 |
| 15 | 16 354 | 38 |
| 21 | 30 125 | 98 |
| 23 | 17 121 | 61 |
| 30 | 6 697 | 31 |
| 31 | 11 046 | 53 |
| 36 | 12 014 | 67 |
| 40 | 5 325 | 33 |
| 43 | 12 103 | 75 |
| 47 | 3 571 | 26 |
| 47 | 13 717 | 100 |
| 50 | 4 034 | 31 |
| 50 | 12 910 | 100 |

Table S5: Small-scale PolyActive membrane data points

| CO2 concentration [mol%] | Gas flow rates [Nm3/h] | Capture scale [kt/y] |
| --- | --- | --- |
| 5 | 13 015 | 10 |
|  | 40 345 | 31 |
|  | 80 690 | 63 |
|  | 130 146 | 100 |
| 6 | 105 411 | 98 |
| 12 | 38 195 | 71 |
| 13.5 | 14 928 | 31 |
| 23 | 16 279 | 58 |
| 30 | 3 228 | 15 |
| 30 | 3 443 | 16 |
| 30 | 6 455 | 30 |
| 35 | 14 202 | 77 |
| 42 | 10 817 | 71 |
| 48 | 4 304 | 32 |
| 48 | 6 859 | 51 |
| 50 | 4 034 | 31 |
| 61 | 4 551 | 43 |
| 64 | 2 730 | 28 |
| 70 | 2 864 | 32 |
| 70 | 8 069 | 88 |
| 70 | 9 214 | 100 |

# REFERENCES

Wang, Y., 2020. Techno-economic assessment of hybrid post-combustion carbon capture systems in coal-fired power plants and steel plants.

IEAGHG, (2024), Baseline Techno-Economic Assessment of Small-Scale Carbon Capture for Industrial and Power Systems, IEAGHG Technical Report 2024-03.

Zhai, H., 2019. Advanced membranes and learning scale required for cost-effective post-combustion carbon capture. Iscience 13, 440–451.