Table 1 summarizes the kinetic data and homogeneous reactions considered for modeling the gas-phase process

Table 1 -  Kinetics of homogeneous reactions

| m | Reaction | Reaction Rate (kmol/m3s) - rm |  | Ref |
| --- | --- | --- | --- | --- |
| 1 | C10H8→10 Csuie+4H2 | r1=k1C10H82H2-0.7 | k1=5.56∙1015∙exp-3.6∙105RT | [1] |
| 2 | C10H8+4H2O→C6H6+4CO+5H2 | r2=k2C10H8H20.4 | k2=1.74∙1012∙exp-3.24∙105RT | [1] |
| 3 | C7H8+H2→C6H6+CH4 | r3=k3C7H8H20.5 | k3=1.04∙1012∙exp-2.47∙105RT | [1] |
| 4 | C6H6+5H2O→5CO+6H2+CH4 | r4=k4[C6H6] | k4=4.4∙108∙exp-2.2∙105RT | [2] |
| 5 | Csuie+H2O→CO+H2 | r5=k5Csuie[H2O] | k5=3.6∙1012∙exp-3.1∙105RT | [1] |
| 6 | CH4+H2O→CO+3H2 | r6=k6CH4[H2O] | k6=4.56∙1012∙exp-1.77∙105RT | [3] |
| 7 | H2+12O2→H2O | r7=k7H2[O2] | k7=1.08∙1010∙exp-1.255∙105RT | [4] |
| 8 | CO+12O2→CO2 | r8=k8COO20.25H2O0.5 | k8=3.165∙1012∙exp-1.8∙105RT | [5] |
| 9 | CO+H2O→CO2+H2 | r9=k9CO[H2O] | k9=2.778∙102∙exp-1.256∙104RT | [5] |
| 10 | CO2+H2→CO+H2O | r10=k10CO2[H2] | k10=1.263∙104∙exp-4.729∙104RT | [5] |
| 11 | CH4+ 2O2→CO2+2H2O | r11=k12CH4-0.3O21.3 | k11=1.3∙105∙exp-2.026∙105RT | [4] |
| 12 | CH4+ 12O2→CO+2H2 | r12=k12CH40.7O20.8 | k12=4.996∙1013∙exp-2.026∙105RT | [5] |
| 13 | C6H6+ 152O2→6 CO2+3H2O | r13=k13C6H6-0.1O21.25 | k13=1.783∙101∙exp-1.255∙105RT | [4] |
| 14 | C6H6+3O2→6CO+3H2 | r14=k14C6H6[O2] | k14=1.58∙1015∙exp-2.026∙105RT | [5] |
| 15 | C7H8+9O2→7 CO2+4 H2O | r15=k15C7 H8-0.1O21.85 | k15=1.6∙1014∙exp-1.2553∙105RT | [4] |
| 16 | C6H6O+4O2→3H2O+6 CO | r16=k16C6H6O-0.1O21.85 | k16=2.4∙1011∙exp-1.2552∙105RT | [6] |
| 17 | C6H6O→CO+0.4 C10H8+0.15 C6H6+ 0.1 CH4+0.75 H2 | r17=k17[C6H6O] | k17=1∙107∙exp-1∙105RT | [6] |
| 18 | C6H6O+3H2O→2CO+CO2+2.95 CH4+0.05 Csuie+0.1 H2 | r18=k18[C6H6O] | k18=2.34∙108∙exp-1∙105RT | [6] |
| 19 | 2CH4→C2H2+3H2 | r19=k19CH4 | k19=3.65∙1011∙exp-3.32∙105RT | [7] |
| 20 | CH4→Csuie+2H2 | r20=k20[CH4]T | k20=2.05∙exp-6.7716∙104RT | [8] |
| 21 | C2H2→2 Csuie+H2 | r22=k22C2H22T2 | k21=13.132∙exp-7.3986∙104RT | [8] |

[1]	A. Lasa-Benito, « Mechanisms and kinetics of thermal reactions of aromatic hydrocarbons from pyrolysis of solid fuels », Fuel., vol. 75, no 12, 1996.

[2]	F. Nozahic, « Production de gaz de synthèse par interactions à haute température du gaz, des goudrons et du résidu carboné issus de la pyrolyse de biomasses », PhD Thesis, 2008. [En ligne]. Disponible sur: http://www.theses.fr/2008INPT006G

[3]	P. S. Virk, L. E. Chambers, et H. N. Woebcke, « THERMAL HYDROGASIFICATION OF AROMATIC COMPOUNDS. », Adv Chem Ser, no 13. p. 237‑258, 1974. doi: 10.1021/ba-1974-0131.ch014.

[4]	S. R. Turns, An Introduction to Combustion: Concepts and Applications. in McGraw-Hill series in mechanical engineering. McGraw-Hill, 1996. [En ligne]. Disponible sur: https://books.google.fr/books?id=rzo8PgAACAAJ

[5]	I. Petersen et J. Werther, « Experimental investigation and modeling of gasification of sewage sludge in the circulating fluidized bed », Chemical Engineering and Processing: Process Intensification, vol. 44, no 7, p. 717‑736, juill. 2005, doi: 10.1016/j.cep.2004.09.001.

[6]	Y. Su, Y. Luo, Y. Chen, W. Wu, et Y. Zhang, « Experimental and numerical investigation of tar destruction under partial oxidation environment », Fuel Processing Technology, vol. 92, no 8, p. 1513‑1524, août 2011, doi: 10.1016/j.fuproc.2011.03.013.

[7]	J. E. Anderson et L. K. Case, « An Analytical Approach to Plasma Torch Chemistry », Ind. Eng. Chem. Proc. Des. Dev., vol. 1, no 3, p. 161‑165, juill. 1962, doi: 10.1021/i260003a001.

[8]	J. Happel et L. Kramer, « ACETYLENE AND HYDROGEN FROM THE PYROLYSIS OF METHANE », Ind. Eng. Chem., vol. 59, no 1, p. 39‑50, janv. 1967, doi: 10.1021/ie50685a008.