Supplementary material for: Integrated molecular and process design with technoeconomic and lifecycle assessment for solvent-based recycling of end-of-life vehicle plastics

Riccardo Standisha, Jian Yinb, Jakob Burgerc, Mirjana Mincevab, Hannah Mangoldd, Christian Holtzed, Markus Schoernerd, Bernhard von Vacanod, Amparo Galindoa, George Jacksona, Claire S. Adjimana,*

a Department of Chemical Engineering, Sargent Centre for Process Systems Engineering, Imperial College London, UK

b Technical University of Munich, TUM School of Life Sciences

c Technical University of Munich, Campus Straubing for Biotechnology and Sustainability

d BASF SE, Carl-Bosch-Strasse 38, 67056 Ludwigshafen am Rhein, Germany

* Corresponding Author: c.adjiman@imperial.ac.uk.

Table S1: Molecular weight and melting properties of the polymers of interest [1]. [*] denotes the property is measured in our current work.

|  | PE | PP | POM |
| --- | --- | --- | --- |
| Mw/(kg mol-1) | 4.91 | 12.00 | 10.00 |
| Tifus/K | 375.15[*] | 430.15[*] | 433.15[*] |
| ∆hifus/(J g -1) | 293.00[1] | 206.7[1] | 326.05[1] |

TEA and Filter methodology

For the capital expenditure (CAPEX) estimation, equipment purchase cost correlations were obtained from Woods et al. [2], as they provide detailed costing guidelines specific to unit operations used in plastic recycling. To estimate the total fixed capital investment, the factorial method was applied to the equipment purchase costs. A total installation factor of 6.05 was selected, corresponding to a fluids-solids process type as outlined in Table 7.5 of Towler and Sinnott [3]. Finally, the discounted cash flow analysis for the MSP and internal rate of return (IRR) assumed a construction period of three years, a seven-year MACRS depreciation schedule, 5% working capital, and a corporate tax rate of 21% [3]. The OPEX and CAPEX contributions to the TAC are given in Table S2. All assumptions used for the technoeconomic analysis are given in Table S3.

The cost of filters depends on the filtration area [2]. It is calculated using the methodology described in Wakeman and Tarleton [11] and is proportional to the viscosity of the filtrate. This is calculated using the temperature-dependent viscosity of the polymer+solvent mixture, assuming a dilute regime, as per the correlations in the HSPiP guide [12]. The solvent viscosities were determined using group contribution methods [13].

LCA Methodology

For utilities and infrastructure, Ecoinvent data were utilized for medium voltage power, steam for chemical processes, factory erection, pumps, pre-processing [9] and the disposal of hazardous chemicals via incineration. Cooling water impacts were modeled using the inventory provided by Schulze et al. [14]. Equipment material inputs were assumed to be carbon steel for process units and polyester fabric for filters, with the latter being replaced every two years. The environmental impacts were evaluated using the GWP 2007, CED, EI99 Endpoint, ReCiPe Endpoint, and the 18 ReCiPe 2016 Midpoint methods.

Solvent cost methodology

Economic data for the solvents was compiled using a hybrid approach combining market literature with machine learning predictions. Bulk pricing for 17 common solvents (including acetone, benzene, and ethyl acetate) was retrieved directly from commercial chemical intelligence sources such as ChemAnalyst [15], Intratec [16], and ICIS [17]. Cymene’s price was obtained from a TEA analysis by Davila et al. [18]. For the remaining 18 solvents lacking transparent market data, prices were estimated using a Random Forest regression model. This algorithm was selected for its robustness in handling small datasets, non-linearities, and feature interactions. The model was trained on a 70/30 split of the known pricing data, using lab-scale costs (Sigma Aldrich [19]), molecular weight, and boiling point as predictive features to generate comparable industrial tonnage estimates.

Objective function normalization

The multi-objective optimization is resolved by minimizing a scalarized objective function derived via the weighted sum method [20], which aggregates Total Annual Cost (TAC) and the ReCiPe Endpoint indicator. To prevent numerical bias and ensure equal weighting, each objective is normalized using Utopia and Nadir values established by initially minimizing the TAC and ReCiPe objectives individually to define the feasible range. The specific minimization problem is defined as:

min F =TAC - 0.31770.4349 - 0.3177+ReCiPe - 9.6421.50 - 9.64

Table S2: OPEX and CAPEX contributions to TAC per unit of processed plastic waste for the top 5 and bottom 5 solvent combinations, and the sum of filter costs S1-S5.

| Label | Solvent 1 + Solvent 2 | TAC/M$/y | OPEX/M$/y | CAPEX/M$/y | Total filter cost/k$ |
| --- | --- | --- | --- | --- | --- |
| S1 | cymene+cyclohexanone | 2.90 | 1.14 | 1.29 | 5.28 |
| S2 | ethylbenzene+cyclohexanone | 2.90 | 1.14 | 1.28 | 5.11 |
| S3 | cymene+decane | 2.91 | 1.14 | 1.29 | 5.46 |
| S4 | cymene+mesitylene | 2.90 | 1.14 | 1.29 | 5.28 |
| S5 | cymene+undecane | 2.91 | 1.14 | 1.29 | 5.50 |
| S68 | dibutoxymethane+octanol | 3.27 | 1.23 | 1.49 | 7.93 |
| S69 | undecane+octanol | 3.32 | 1.22 | 1.53 | 8.88 |
| S70 | undecane+decanol | 3.33 | 1.22 | 1.54 | 9.00 |
| S71 | dodecane+octanol | 3.35 | 1.23 | 1.55 | 9.08 |
| S72 | dodecane+decanol | 3.35 | 1.23 | 1.56 | 9.20 |

Table S3: Summary of TEA parameters, assumptions, costs, and their references.

| Parameter | Value |
| --- | --- |
| Lifetime of plant (years) [4] | 20 |
| Operating hours (hours/year) | 8000 |
| Plastic waste feed (ton/year) | 3000 |
| Composition of plastic waste feed | 45 % LDPE, 45% PP, 10% POM |
| Base year | 2025 |
| ISBL costs [3] | 320% of the equipment purchase cost |
| OSBL costs [3] | 40% of ISBL |
| Engineering costs [3] | 25% of ISBL+OSBL |
| Contingency cost [3] | 10% of ISBL+OSBL |
| Working Capital [3] | 5% of CAPEX |
| Depreciation method & recovery period | 7-year MACRS |
| Construction period (years) [5] and spending schedule | 3 (8% Y1, 60% Y2, 32%, Y3) |
| Corporate tax rate [3] | 21% |
| Internal rate of return (IRR) (%) [5] | 20% |
| Operator salary ($/year); number of operators per shift position; number of shift positions [6] | 65862; 2; 3 |
| Maintenance [3] | 3% of ISBL |
| Plant overhead [3] | 65% of maintenance and 117.8125% of salaries |
| Tax and insurance [3] | 2% of ISBL+OSBL |
| Rent of land [3] | 1% of ISBL+OSBL |
| Pre-processing cost [7,8] ($/s) | 0.0039F10.65627+0.055F1 |
| Incineration cost ($/kg) [9] | 0.00406 |
| PP price ($/kg) [10] | 1.50 |
| LDPE price ($/kg) [10] | 1.71 |
| ]POM price ($/kg) [10] | 2.55 |
| Electricity ($/kWh) [3] (adjusted for inflation) | 0.06 |
| Natural gas ($/MMBtu) [3] (adjusted for inflation) | 6.92 |
| Boiler efficiency [3] | 80% |
| Turbine efficiency [3] | 85% |
| Cost of cooling water makeup and chemical treatment ($/gal) [3] (adjusted for inflation) | 0.00002 |
| Power consumption cooling tower (kWh/1000gal) [3] | 1.5 |
| PP extrusion energy consumption (kWs/g) [2] | 1 |
| LDPE extrusion energy consumption (kWs/g) [2] | 0.9 |

Table S3: References for raw materials, utilities, and products substituted in the LCA.

| Component | Component in Ecoinvent |
| --- | --- |
| Heat from steam | Steam production, as energy carrier, in chemical industry |
| Electricity | Market for electricity, low voltage |
| Tap water | Market for tap water, Rest of the World |
| Steel | Steel, chromium steel 18/8, hot rolled |
| Incineration | Treatment of spent solvent mixture, hazardous waste incineration, spent solvent mixture |
| Polyester filter | Market for textile, non-woven polyester, textile, non-woven polyester |
| Factory | Chemical factory construction, organics |
| FeCl3 | iron(III) chloride, without water, in a 12% iron solution state |
| Maize starch | Market for maize starch, rest of world |
| NaOH | Sodium hydroxide, without water, in 50% solution state |

References

B. Wunderlich et al. Thermodynamic Properties of Polymers. Encyclopedia of Polymer Science and Engineering 16:767–807 (1989)

Woods, D.R., 2007. Rules of Thumb in Engineering Practice. Wiley-VCH,943 Weinheim.

Towler G, Sinnott R. Economic evaluation of projects. In: Chemical Engineering Design. Elsevier (2022)

Targett PNM, Naviroj P, Treacy J, Urffer C, Riggleman RA. https://repository.upenn.edu/entities/publication/d0de6f8d-3e41-4018-9c07-fb752224de11

Chaudhari US, et al. Solvent based dissolution–precipitation of waste polyethylene terephthalate: Economic and environmental performance metrics. RSC Sustain 1:1849–1860 (2023) https://doi.org/10.1039/D3SU00231D

K. L. Sánchez-Rivera et al., ‘Reducing Antisolvent Use in the STRAP Process by Enabling a Temperature-Controlled Polymer Dissolution and Precipitation for the Recycling of Multilayer Plastic Films’, ChemSusChem, vol. 14, no. 19, pp. 4317–4329, 2021, doi: 10.1002/cssc.202101128.

M. Larrain et al., ‘Techno-economic assessment of mechanical recycling of challenging post-consumer plastic packaging waste’, Resources, Conservation and Recycling, vol. 170, p. 105607, July 2021, doi: 10.1016/j.resconrec.2021.105607.

B. Hernández, D. G. Vlachos, and M. G. Ierapetritou, ‘Superstructure optimization for management of low-density polyethylene plastic waste’, Green Chem., vol. 26, no. 17, pp. 9476–9487, Aug. 2024, doi: 10.1039/D4GC00339J.

Burhan S, Srocka M, Ciroth A, Recanati F. ecoinvent v.3.6 in openLCA. (2019)

Businessanalytiq. https://businessanalytiq.com/

E. S. Tarleton and R. J. Wakeman. 7 - Process design for continuous separations. In: Solid/Liquid Separation. Ed: Butterworth-Heinemann. Oxford (2007).

Hansen, C. M. Hansen solubility parameters: a user’s handbook ; CRC press, 2007.

H.-C. Hsu, Y.-W. Sheu, and C.-H. Tu, ‘Viscosity estimation at low temperatures (Tr<0.75) for organic liquids from group contributions’, Chemical Engineering Journal, vol. 88, no. 1, pp. 27–35, Sept. 2002, doi: 10.1016/S1385-8947(01)00249-2.

Schulze C, Thiede S, Herrmann C. Life cycle assessment of industrial cooling towers. In: Progress in Life Cycle Assessment. Ed: Schebek L, Herrmann C, Cerdas F. Springer International Publishing (2019)

Chemanalyst. https://www.chemanalyst.com/

Intratec. https://www.intratec.us/

ICIS. https://www.icis.com/

J. A. Dávila, M. Rosenberg, and C. A. Cardona. Techno-economic and Environmental Assessment of p-Cymene and Pectin Production from Orange Peel. Waste Biomass Valor, 6:253–261 (2015) 10.1007/s12649-014-9339-y.

Sigma Aldrich. https://www.sigmaaldrich.com/

Lee, Y.S., Graham, E.J., Galindo, A., Jackson, G., Adjiman, C.S., 2020. A comparative study of multi-objective optimization methodologies for molecular and process design. Computers & Chemical Engineering 136, 106802. https://doi.org/10.1016/j.compchemeng.2020.106802

© 2026 by the authors. Licensed to PSEcommunity.org and PSE Press. This is an open access article under the creative commons CC-BY-SA licensing terms. Credit must be given to creator and adaptations must be shared under the same terms. See https://creativecommons.org/licenses/by-sa/4.0/