Particle Swarm Optimization for simultaneous design and optimization of heat pumps considering Mixed Integer problems- Supporting Information

Beatriz C. da Silva a*, Ana M. Ribeiro a, Alírio E. Rodrigues a, Alexandre F.P. Ferreira a, Diogo Rodrigues a, Idelfonso B.R. Nogueira b

a LSRE-LCM, ALiCE, Faculty of Engineering, University of Porto, Rua Dr. Roberto Frias, 4200-465 Porto, Portugal

b Chemical Engineering Department, Norwegian University of Science and Technology, Sem Sælandsvei 4, Kjemiblokk 5, Trondheim 7491, Norway

* Corresponding Author: up201707213@fe.up.pt.

Base_PSO algorithm

To start the PSO, one must define the number of particles to be evaluated in each iteration (Np), the maximum number of iterations to be performed (Nit) and the side constraints for each variable (Xmin and Xmax).

In the algorithm, the particle positions (X) and velocities (V) are randomly initialized. The positions are evaluated to assess the best position in the swarm (Yglobest). The particle position is updated according to Equation 1, where xi,n,k* is the candidate position for the variable n, particle i, and iteration k, vi,n, k is the velocity used to calculate the new position and xi,n,k-1 is the previous position of the particle.

| xn,i,k*=vi,n, k+xi,n,k-1 | (1) |
| --- | --- |

After the initial velocity being randomly determined for starting the procedure, the following velocities (vi,n,k for k>1) are calculated resorting to Equation 2.

| vi,n,k=Covi,n,k-1+C1r1Xglobest-xi,n,k-1+C2r2(Xpbest-xi,n,k-1) | (2) |
| --- | --- |

where  Xglobest is the position corresponding to Yglobest so far, Xpbest  is the best position ever achieved by the particle i  , r1 and r2 are two random numbers between 0 and 1, Co, C1 (2) and C2 (2) are acceleration coefficients, and  k is the current iteration. Co is variable throughout the optimization, according to Equation 3, where Co,f  (0.4) and C0,i (0.9) are the final and initial values of Co (Co,i>C0,f).

| Co=C0,i+ k-1Nit-1 (Co,f-C0,i) | (3) |
| --- | --- |

With the candidate position calculated, the upper and lower bounds of the variables are enforced. In case a given particle goes beyond the established limits, violation rates (wL and wU) are calculated through Equations 4 and 5, aiming to quantify which bound has the higher surpassing.

| wL=maxnXmin-xi,n,k*xi,n,k-1-Xmin | (4) |
| --- | --- |
| wU=maxnxi,n,k*-XmaxXmax-xi,n,k-1 | (5) |

Then, the new particle position is determined by Equation 6, where w is the maximum violation rate (w=max(wL, wU)).

| xi,n,k=xi,n,k-1+xi,n,k*-xi,n,k-11+w | (6) |
| --- | --- |

When the final iteration is completed, the global optimum of the swarm is considered the global optimum of the problem.

The Base_PSO algorithm is depicted in Figure 1. When used for integer variables, this algorithm resorts to a rounding operation through the traditional rounding operator of the considered software. The constraints were enforced by a penalization of the objective function.

Figure 1. Particle Swarm Optimization algorithm.

References:

- 1.	Nogueira, I.B.R., et al., Optimization of a True Moving Bed unit and determination of its feasible operating region using a novel Sliding Particle Swarm Optimization. Computers & Industrial Engineering, 2019. 135: p. 368-381. https://doi.org/10.1016/j.cie.2019.06.020.

- 2.	Rebello, C.M., et al., From an Optimal Point to an Optimal Region: A Novel Methodology for Optimization of Multimodal Constrained Problems and a Novel Constrained Sliding Particle Swarm Optimization Strategy. Mathematics, 2021. 9(15). https://doi.org/10.3390/math9151808.

- 3.	Rebello, C.M., et al., From a Pareto Front to Pareto Regions: A Novel Standpoint for Multiobjective Optimization. Mathematics, 2021. 9(24). https://doi.org/10.3390/math9243152.

- 4.	Nogueira, I.B.R., et al., A novel nested loop optimization problem based on deep neural networks and feasible operation regions definition for simultaneous material screening and process optimization. Chemical Engineering Research and Design, 2022. 180: p. 243-253. https://doi.org/10.1016/j.cherd.2022.02.013.

- 5.	Rebello, C.M., et al., A novel standpoint of Pressure Swing Adsorption processes multi-objective optimization: An approach based on feasible operation region mapping. Chemical Engineering Research and Design, 2022. 178: p. 590-601. https://doi.org/10.1016/j.cherd.2021.12.047.

- 6.	Silva, B.C., et al., Metaheuristic Framework for Material Screening and Operating Optimization of Adsorption-Based Heat Pumps. ACS Omega, 2023. 8(22): p. 19874-19891. https://doi.org/10.1021/acsomega.3c01797.