Supplementary material for: Estimation of Thermodynamic Properties for Cellulosic Biomass-Derived Compounds: Application to Heat and Work Balances in Process Simulation

Anthony D. Anastasia*, Cornelius M. Masukub, Praveen Ravikumara, Shishir P.S. Chundawata, and Diane Hildebrandta

a Rutgers, The State University of New Jersey, Department of Chemical and Biochemical Engineering, Piscataway, NJ, USA

b Purdue University, Davidson School of Chemical Engineering, West Lafayette, IN, USA

* Corresponding Author: ada149@rutgers.edu

model ACcess

Preliminary model will be found on GitHub at: https://github.com/tonyanastasi22/biomass-thermo-model. If multiple models are available, as the process is further defined, version v1.0 (Thermo-Model v1.0 in packages) was used to produce results for this paper. Further models may provide more accurate data as more limitations are relaxed with regards to other compounds or increasing accuracy of entropy calculations.

model limitations

This work presents a regression-based thermodynamic framework for estimating combustion enthalpy (∆HC), Gibbs free energy (∆GC) and formation properties (∆HF, ∆GF, ∆SF, S0) of CHO-based organic compounds. The core of the model is a linear relationship between ∆HC and the compound’s molar oxygen requirement, υO2, consistent with Thornton’s rule. Formation properties are derived from empirical regressions of elemental composition (C, H, O).

The model was trained on 50+ organic compounds and validated against reported experimental values of biomass materials, showing low deviations for biomass constituents.

However, entropy-related predictions are inherently more sensitive due to their derivation from∆HF and ∆GF. Small inaccuracies in these values can lead to amplified uncertainty in entropy estimations. Additionally, this model has not been calibrated for oxygen-rich or low-carbon compounds (υO2< 0.5), and such cases may return unreliable or non-physical results.

This version of the model should be used as a lightweight tool for first-pass thermodynamic screening in process modeling or bioenergy system design. Future improvements will focus on refining regressions using expanded datasets and potentially incorporating additional features. Additionally, the impact of nitrogen and sulfur atoms, as are found in many organic biomass compounds, will potentially be examined and incorporated into future models.

Table S1: Thermodynamic properties retrieved from NIST WebBook [1].

| Family | Compound | Formula | ∆HF kJ mol-1 | S0 J mol-1 K-1 | ∆SF J mol-1 K-1 | ∆GF kJ mol-1 |
| --- | --- | --- | --- | --- | --- | --- |
| Alkanes | Methane | CH4 | -74.87 | 186.25 | -80.94 | -50.74 |
|  | Ethane | C2H6 | -84.00 | 229.6 | -174.11 | -32.09 |
|  | Propane | C3H8 | -104.70 | 269.91 | -270.31 | -24.11 |
|  | Butane | C4H10 | -125.60 | 310.1 | -366.63 | -16.29 |
|  | Pentane | C5H12 | -146.80 | 347.82 | -465.43 | -8.03 |
|  | Hexane | C614 | -198.70 | 296.06 | -653.70 | -3.80 |
|  | Heptane | C7H16 | -224.40 | 328.57 | -757.70 | 1.51 |
|  | Octane | C8H18 | -250.30 | 361.2 | -861.58 | 6.58 |
|  | Nonane | C9H20 | -274.70 | 393.67 | -965.63 | 13.20 |
|  | Decane | C10H22 | -301.00 | 425.89 | -1069.92 | 18.00 |
|  | Undecane | C11H24 | -327.20 | 458.12 | -1174.20 | 22.89 |
|  | Dodecane | C12H26 | -352.10 | 490.66 | -1278.18 | 28.99 |
| Branched Alkanes | 2-Methylpropane | C4H10 | -134.20 | 290 | -386.73 | -18.90 |
|  | 2-Methylbutane | C5H12 | -178.20 | 260.41 | -552.84 | -13.37 |
|  | 2-Methylpentane | C7H16 | -204.30 | 290.58 | -795.69 | 32.94 |
|  | 2-Methylhexane | C8H18 | -229.80 | 323.34 | -899.44 | 38.37 |
|  | 2-Methylheptane | C9H20 | -255.20 | 356.39 | -1002.91 | 43.82 |
|  | 2,2-Dimethylpropane | C5H12 | -167.90 | 306 | -507.25 | -16.66 |
| Cyclohexanes | Cyclopropane | C3H6 | 53.30 | 237.4 | -172.14 | 104.62 |
|  | Cyclopentane | C5H10 | -105.60 | 204.14 | -478.43 | 37.04 |
|  | Cyclohexane | C6H12 | -157.70 | 203.89 | -615.19 | 25.72 |
| Alkenes | Ethene | C2H4 | 52.47 | 219.32 | -53.71 | 68.48 |
|  | Propene | C3H6 | 20.41 | 266.82 | -142.72 | 62.96 |
|  | But-1-ene | C4H8 | -0.63 | 307 | -239.05 | 70.64 |
|  | trans-But-2-ene | C4H8 | -10.8 | 209 | -337.05 | 89.69 |
|  | cis-But-2-ene | C4H8 | -7.7 | 209 | -337.05 | 92.79 |
|  | Buta-1,2-diene | C4H6 | 162.2 | 321.21 | -94.16 | 190.27 |
|  | Buta-1,3-diene | C4H6 | 108.8 | 278.78 | -136.59 | 149.52 |
| Alkynes | Ethyne | C2H2 | 226.73 | 200.93 | 58.58 | 209.26 |
|  | Propyne | C3H4 | 185.4 | 268.6 | -10.26 | 188.46 |
|  | 1-Butyne | C4H6 | 165.2 | 310.23 | -105.14 | 196.55 |
|  | 2-Butyne | C4H6 | 145.1 | 195.1 | -220.27 | 210.77 |

Table S1 cont.

| Family | Compound | Formula | ∆HF kJ mol-1 | S0 J mol-1 K-1 | ∆SF J mol-1 K-1 | ∆HF kJ mol-1 |
| --- | --- | --- | --- | --- | --- | --- |
| Arenes | Benzene | C6H6 | 49 | 173.26 | -253.78 | 124.66 |
|  | Methylbenzene | C7H8 | 12 | 220.96 | -342.59 | 114.14 |
|  | Ethylbenzene | C8H10 | -12.5 | 255.01 | -445.05 | 120.19 |
|  | Propylbenzene | C9H12 | -38.4 | 287.78 | -548.80 | 125.22 |
|  | 1,2-Dimethylbenzene | C8H10 | -24.4 | 246.02 | -454.04 | 110.97 |
|  | 1,3-Dimethylbenzene | C8H10 | -25.4 | 253.8 | -446.26 | 107.65 |
|  | 1,4-Dimethylbenzene | C8H10 | -24.4 | 247.154 | -452.91 | 110.64 |
| Alcohols | Methanol | CH4O | -238.4 | 127.19 | -242.58 | -166.08 |
|  | Ethanol | C2H6O | -276 | 159.86 | -346.42 | -172.71 |
|  | Propan-1-ol | C3H8O | -302.54 | 192.8 | -449.99 | -168.37 |
|  | Propan-2-ol | C3H8O | -317 | 180.58 | -462.21 | -179.19 |
|  | Butan-1-ol | C4H10O | -328 | 225.73 | -553.58 | -162.95 |
|  | Pentan-1-ol | C5H12O | -351.6 | 258.9 | -656.92 | -155.74 |
|  | Hexan-1-ol | C6H14O | -377.5 | 287.4 | -764.93 | -149.44 |
|  | Heptan-1-ol | C7H16O | -403.4 | 325.9 | -862.95 | -146.11 |
|  | Octan-1-ol | C8H18O | -426.6 | 355.9 | -969.46 | -137.56 |
|  | Ethan-1,2-diol | C2H6O2 | -460 | 166.9 | -441.96 | -328.23 |
| Carboxylic acids | Formic Acid | HCOOH | -425.5 | 131.84 | -209.82 | -362.94 |
|  | Acetic Acid | CH3COOH | -483.52 | 158 | -320.18 | -388.06 |
|  | Oxalic Acid | (COOH)2 | -828.93 | 115.6 | -437.05 | -698.62 |
|  | Benzoic Acid | C6H5COOH | -384.8 | 165.71 | -472.31 | -243.98 |

Table S2: Combustion reaction data for data retrieved from NIST WebBook [1].

| Compound | Formula | ∆HF kJ mol-1 | ∆GF kJ mol-1 | ∆HC kJ mol-1 | ∆GC kJ mol-1 |
| --- | --- | --- | --- | --- | --- |
| Methane | CH4 | -74.87 | -50.74 | 890.30 | 817.90 |
| Ethane | C2H6 | -84.00 | -32.09 | 1560.51 | 1468.05 |
| Propane | C3H8 | -104.70 | -24.11 | 2219.15 | 2107.53 |
| Butane | C4H10 | -125.60 | -16.29 | 2877.59 | 2746.85 |
| Pentane | C5H12 | -146.80 | -8.03 | 3535.73 | 3386.61 |
| Hexane | C6H14 | -198.70 | -3.80 | 4163.17 | 4022.34 |
| Heptane | C7H16 | -224.40 | 1.51 | 4816.81 | 4659.15 |
| Octane | C8H18 | -250.30 | 6.58 | 5470.25 | 5295.72 |
| Nonane | C9H20 | -274.70 | 13.20 | 6125.19 | 5933.84 |
| Decane | C10H22 | -301.00 | 18.00 | 6778.23 | 6570.14 |
| Undecane | C11H24 | -327.20 | 22.89 | 7431.37 | 7206.53 |
| Dodecane | C12H26 | -352.10 | 28.99 | 8085.81 | 7844.13 |
| 2-Methylpropane | C4H10 | -134.20 | -18.90 | 2868.99 | 2744.24 |
| 2-Methylbutane | C5H12 | -178.20 | -13.37 | 3504.33 | 3381.27 |
| 2-Methylpentane | C7H16 | -204.30 | 32.94 | 4836.91 | 4690.58 |
| 2-Methylhexane | C8H18 | -229.80 | 38.37 | 5490.75 | 5327.51 |
| 2-Methylheptane | C9H20 | -255.20 | 43.82 | 6144.69 | 5964.46 |
| 2,2-Dimethylpropane | C5H12 | -167.90 | -16.66 | 3514.63 | 3377.98 |
| Cyclopropane | C3H6 | 53.30 | 104.62 | 2091.32 | 1999.12 |
| Cyclopentane | C5H10 | -105.60 | 37.04 | 3291.10 | 3194.54 |
| Cyclohexane | C6H12 | -157.70 | 25.72 | 3918.34 | 3814.72 |
| Ethene | C2H4 | 52.47 | 68.48 | 1411.15 | 1331.48 |
| Propene | C3H6 | 20.41 | 62.96 | 2058.43 | 1957.46 |
| But-1-ene | C4H8 | -0.63 | 70.64 | 2716.73 | 2596.64 |
| trans-But-2-ene | C4H8 | -10.80 | 89.69 | 2706.56 | 2615.69 |
| cis-But-2-ene | C4H8 | -7.70 | 92.79 | 2709.66 | 2618.79 |
| Buta-1,2-diene | C4H6 | 162.20 | 190.27 | 2593.73 | 2479.13 |
| Buta-1,3-diene | C4H6 | 108.80 | 149.52 | 2540.33 | 2438.38 |
| Styrene | C8H8 | 103.40 | 201.46 | 4394.80 | 4304.90 |
| Ethyne | C2H2 | 226.73 | 209.26 | 1299.58 | 1235.12 |
| Propyne | C3H4 | 185.40 | 188.46 | 1937.59 | 1845.82 |

References

Linstrom, P.J., Mallard, W.G. NIST Chemistry WebBook. In: NIST Standard Reference Database Number 69. Ed: Linstrom, P.J., Mallard, P.J. National Institute of Technology.

© by the authors. Licensed to PSEcommunity.org and PSE Press. This is an open access article under the creative commons CC-BY-SA licensing terms. Credit must be given to creator and adaptations must be shared under the same terms. See https://creativecommons.org/licenses/by-sa/4.0/